GenApi Tutorial

- Work in Progress -

Table of Contents

1	Overview	3

2	Getting Started	3

2.1	Hello World!	3

2.2	Hello World - Again	4

2.3	Hello World – and Again	5

2.4	Fehler! Kein gültiger Dateiname.First Connect	7

2.5	Minimum and Maximum	9

2.6	Minimum and Maximum Extended with Logging	11

2.7	Dealing with Bit Fields, Enumerations, and Commands	12

2.8	XML Formulas	14

2.9	The Feature Tree and Callbacks	16

History

| Version | Date | Changed by | Change |
| --- | --- | --- | --- |
| 0.1 | 29.04.2010 | Fritz Dierks, Basler | Started the document |
| 0.1 | 02.05.2010 | Fritz Dierks, Basler | Finished GettingStarted section |
| 0.2 | 24.05.2010 | Fritz Dierks, Basler | Extended the GettingStarted section |

# Overview

This document describes how to use the GenApi reference implementation under MS Windows. It is assumed that you are familiar with C++ and VisualStudio, and have a basic understanding of GenICam e.g. by having read the standard document.

# Getting Started

## Hello World!

First we create the most basic GenApi program "Hello World". You need a Visual C++ compiler version VC71 or higher.

Install the GenICam Runtime and SDK. Download the most recent version of the GenICam GenApi Reference Implementation from www.genicam.org. Run the installer using the installation type Test Installation.

The installer will copy the GenICam files to c:/program files/GenICam_v2_1. In addition it will create several environment variables. The most important one is GENICAM_ROOT_V2_1 which identifies the location GenICam was installed.

Start VisualStudio and create a new Win32 Console Application with pre-compiled headers named HelloWorld.

Go to the project's properties, set the configuration to All Configurations, and add $(GENICAM_ROOT_V2_1)/library/CPP/Include as additional include directory and $(GENICAM_ROOT_V2_1)/library/CPP/Lib/Win32_i86 as additional library directory. Note that VisualStudio understands the $() notion so you do not have to replace the environment variable manually.

Open the file StdAfx.h and add the statement #include "GenApi/GenApi.h". This is everything you need in order to use GenICam. The necessary libraries are linked in via #pragma linkage statements provided via GenApi.h.

Now add a file HelloWorld.xml to the project with the following content:

<?xml version="1.0" encoding="utf-8"?>

<RegisterDescription

ModelName="MyVendor"

VendorName="MyCamera"

ToolTip="A very fast and powerful GigE area scan camera"

StandardNameSpace="GEV"

SchemaMajorVersion="1"

SchemaMinorVersion="1"

SchemaSubMinorVersion="0"

MajorVersion="1"

MinorVersion="0"

SubMinorVersion="0"

ProductGuid="2D932CC6-EB68-40bd-B6CC-F03B55B7D653"

VersionGuid="02A8C268-BEE8-463b-A6C0-53ED8256E3D8"

xmlns="http://www.genicam.org/GenApi/Version_1_1"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xsi:schemaLocation="http://www.genicam.org/GenApi/Version_1_1

http://www.genicam.org/GenApi/GenApiSchema_Version_1_1.xsd">

<String Name="TheNode" >

<Value>Hello World!</Value>

</String>

</RegisterDescription>

This XML file contains an outer enclosing node <RegisterDescription> with some attributes describing the camera. In addition it contains a single node which does nothing but hold a string “Hello World!”. New name of the node is TheNode and its type is String. Since a GenICam XML file is just a collection of GenICam nodes the file’s content can be visualized by the following graph:

Now add some code to HelloWorld.cpp:

#include "stdafx.h"

#include <iostream>

using namespace std;

using namespace GenICam;

using namespace GenApi;

int _tmain(int argc, _TCHAR* argv[])

{

CNodeMapRef Camera;

Camera._LoadXMLFromFile("HelloWorld.xml");

CStringPtr ptrTheNode = Camera._GetNode("TheNode");

cout << ptrTheNode->ToString() << "\n";

return 0;

}

The using namespaces statements refer to two namespaces defined by GenICam: GenICam for basic types like strings, exceptions, etc. and GenApi for all other GenApi specific symbols.

CNodeMapRef is a smart pointer like helper class used to instantiate a GenApi node map. The method _LoadXMLFromFile is used to create the node map described in HelloWorld.xml.

The next statement retrieves a pointer to the string node with the name "The Node" using the method _GetNode with the node’s name as argument. The is handed over to a smart pointer ptrTheNode of type CStringPtr which gives access to the methods defined for a string as shown in the following Intellisense drop down box:

One of the methods provided for a string node is ToString which converts the node content to a reasonable string representation. This method is used to output the node’s string content which is “Hello World!”.

## Hello World - Again

A slightly extended version of the HelloWorld program can be found in the /Tests subdirectory. In order to compile the tests you must extend your build system as described in Building_and_Releasing.doc which is located in the /Doc subdirectory.

You have two choices: If you are GenICam member get yourself the source code and follow the build steps until your solution compiles and the tests run successfully. If you have access only to the installer you need minimum required tools as described in the document. Make sure to use the very same compiler version as was used to build the installer which is currently VS80. Then run CompileAndRunGenICamTest.cmd from the /Tests subdirectory. GenICam itself does not come with VisualStudio project and solution files but creates these using CMake. The batch file CompileAndRunGenICamTest.cmd runs CMake and creates the project and solution files in a subdirectory Win32_i86_VS80. In this subdirectory you will find the VisualStudio solution file GenICamTest.sln which contains all projects referred to in this document.

In the project HelloWorld's main file HelloWorld.cpp you find the following slightly extended code:

try

{

gcstring XMLFileName = "$(" GENICAM_ROOT_VERSION ")"

"/Tests/GenApi/HelloWorld/HelloWorld.xml";

CNodeMapRef Camera;

Camera._LoadXMLFromFile(XMLFileName);

CStringPtr ptrTheNode = Camera._GetNode("TheNode");

cout << ptrTheNode->ToString() << "\n";

return 0;

}

catch(GenericException &e)

{

cout << "Error " << e.GetDescription() << "\n";

return -1;

}

First of all the XML file name is given as full path with a $() reference to the GenICam root directory.

XMLFileName = "$(GENICAM_ROOT_V2_1)/Tests/GenApi/HelloWorld/HelloWorld.xml"

This reference is automatically resolved by the _LoadXMLFromFile method as will most GenICam methods do.

In addition a try/cach block encloses the whole code, the catch block catching a GenericException which is the base class of all GenICam exceptions and itself inherits from std::exception.

## Hello World – and Again

The author of a programming tutorial inevitably faces the problem how to organize the example code and how to make sure the code compiles and runs successfully all the time. Writing a lot of small stand alone programs quickly leads to a lot of maintenance effort so this tutorial's examples code was organized differently: all pieces of example code are written as CPPUnit based test cases. The advantage is that running the test code successfully guarantees that the examples are still sound. In order to make sure that the code examples in the tutorial text always match the real test code we have developed a so called Snipper tool which extracts parts of the sample code marked by special comments, does some syntax highlighting and provides the extracted code snippets as stand alone RTF file. This file is then included into the tutorial text by reference. Running the Snipper again and updating the reference links will update every code sample in the text. For details about the Snipper see SnipperDoc.doc.

In order to illustrate this the HelloWorld example has been transferred to the GenApiTutorial project which is a CPPUnit DLL. The code can be found in the method GettingStarted::HelloWorld located in the file GettingStarted.cpp.

GenICam provides two ways to run tests from CPPUnit DLLs like the GenApiTutoria.dll: The DLLPlugInRunner is a graphical front end and the DllPluInTester a console application.

For invoking the test via the graphical front end start the DLLPlugInRunner which is located in the bin/Win32_i86 sub directory. The program will open a file dialog box: select the GenApiTutorial DLL and the program will show the following dialog:

You can either run all tests/examples provided in the DLL by pressing the Run button or select a subgroup of tests. The test cases are organized as a tree which can be displayed by pressing the Browse button.

The console alternative DllPluInTester is best invoked from the debugger. Make the GenApiTutorial the startup project in VisualStudio and select the DllPluInTester EXE as Command. For Command Argument choose 
-w "$(TargetPath)" as shown in the screen shot. This will run all tests and pause for a key at the end (the –w option).

If you want to run a single test only, e.g. GettingStarted::HelloWorld extend the Command Arguments with 
:GettingStarted::HelloWorld (note the leading colon).

A special problem faced by the author of a GenICam Tutorial is the necessity of a lot of XML example snippets. The reader of the example code should be able to see at a glance which XML snippet belongs to a codes sample. If each snippet resides in a separate file this can be very tedious.

This problem is again solved with the help of the Snipper tool: The XML code is simply added inside a special comment to the CPP file and extracted by the snipper to an XML file. Have a look at the GettingStarted::HelloWorld method in the file GettingStarted.cpp: The comment holding the XML snippet tooks like this:

void GettingStarted::HelloWorld()

{

/*[ GenApiTutorial@GettingStarted_HelloWorld.xml|gxml|dot

<String Name="TheNode" >

<Value>Hello World!</Value>

</String>

*/

It is extracted to the file /xml/GenApi/GenApiTutorial/GettingStarted_HelloWorld.xml which is encoded in the comment header. Note that the Snipper automatically adds the enclosing <RegisterDescription> node required for each GenICam XML file.

GenICam treats subdirectories to /xml/GenApi in a special manner: it provides a version of the _LoadXMLFromFile method which takes as parameters a VendorName and a ModelName and assumes that VendorName is the name of a subdirectory and ModelName the name of a XML file. So the generated XML file can be loaded using the following code:

CNodeMapRef Camera;

Camera._LoadXMLFromFile("GenApiTutorial", "GettingStarted_HelloWorld");

CStringPtr ptrTheNode = Camera._GetNode("TheNode");

cout << ptrTheNode->ToString() << "\n";

## First Connect

Now we do our first connect to a camera. Assume a camera has a 4 byte integer Gain register at address 0xffff0000. We want to create a XML file which describes this register, connect to a (pseudo) camera, and read and write the Gain register using GenICam.

GenICam comes with a lot of different node types for different purposes. In order to map an integer register we use the <IntReg> node. For connecting the camera we need a <Port> node. The necessary XML code looks like this:

<IntReg Name="GainReg" >

<Address>0xffff0000</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Endianess>LittleEndian</Endianess>

</IntReg>

<Port Name="Device"/>

The <IntReg> node named GainReg contains all necessary information to describe the register: its Address, its Length, and its Endianess. In addition the AccessMode must be given which in our case is RW (= read/write). The <IntReg> node does not directly connect to the camera but to a proxy node of type <Port> named Device. The connection is established by the <pPort>Device</pPort> entry in the <IntReg> node, the 'p' prefix standing for 'pointer to'. Graphically the XML file's content can be expressed like this:

Now we need to connect the <Port> proxy node to the camera. GenICam itself does not provide a driver or transport layer. Instead it requires a IPort interface which lets it read and write to the camera's register space.

The IPort interface inherits from the IBase interface and contains three methods which must be implemented by the client:

For demonstration the following small helper class is provided in GettingStarted.cpp implementing IPort:

// This class provides read/write access to the camera's registers

class CTransportLayer : public IPort

{

public:

virtual EAccessMode GetAccessMode() const

{

// If the driver is open, return RW (= read/write),

// otherwise NA (= not available)

return RW;

}

virtual void Read(void *pBuffer, int64_t Address, int64_t Length)

{

// Fetch <Length> bytes starting at <Address> from the camera

// and copy them to <pBuffer>

// ...this is a simulation only

CPPUNIT_ASSERT_EQUAL( 0xffff0000LL, Address );

CPPUNIT_ASSERT_EQUAL( 4LL, Length );

*((int64_t*)pBuffer) = m_Gain;

}

virtual void Write(const void *pBuffer, int64_t Address, int64_t Length)

{

// Copy <Length> bytes from <pBuffer> to the camera

// starting at <Address>

// ...this is a simulation only

CPPUNIT_ASSERT_EQUAL( 0xffff0000LL, Address );

CPPUNIT_ASSERT_EQUAL( 4LL, Length );

m_Gain = *((int64_t*)pBuffer);

}

Each GenICam node has an AccessMode which indicates if the node is implemented and currently readable and/or writable. In case of a transport layer the AccessMode will typically be RW (= read/write) while the underlying driver is open and NA (= not available) while the driver is closed.

The signatures of the remaining two methods Read and Write are quite similar: they take a pointer to a buffer, an address value, and a length which is the number of bytes to read or write respectively. In the example a member variable int64_t::m_Gain is used to store the Gain's value.

With the CTransportLayer helper class we can now instantiate a pseudo camera and connect it to the node map. This is done using the _Connect method which takes a IPort interface and (optionally) the name of the <Port> node to connect to. By SFNC convention the standard name of a camera port is Device.

CNodeMapRef Camera;

Camera._LoadXMLFromFile("GenApiTutorial", "GettingStarted_FirstConnect");

CTransportLayer TransportLayer;

Camera._Connect( &TransportLayer, "Device" );

CIntegerPtr ptrGain = Camera._GetNode("GainReg");

ptrGain->SetValue(42);

cout << "Gain = " << ptrGain->GetValue() << "\n";

From the HelloWorld example we are already familiar with the IString interface implemented by all nodes which deal with strings. Since the <IntReg> node deals with integers it implements IInteger which has a slightly different set of methods the most important being GetValue and SetValue. Note that in the GenICam reference implementation GenICam all integers are in64 signed which is sufficient for nearly all practical purposes.

It is important to understand the different roles in a real world system so they are shown again in the following diagram: the vendor code provides the transport layer and embeds GenICam in his software. The client then can communicates with the camera through the GenICam nodes.

Port::Device

IntReg::GainReg

Client Code

Transport Layer

Vendor Code

GenICam

IPort interface

## Minimum and Maximum

A client attempting to write generic code accessing the Gain feature of a camera needs to know the Gain's minimum and maximum value. Since these values will differ from camera to camera there must be a way to retrieve them from the camera. Let us assume a cameras Gain range is [0, 100].

The following example shows how the range values can be hard coded in the XML file. For that purpose we use a node of type <Integer> named Gain which takes its value via the <pValue> link from the GainReg node. In addition it has a <Min> and a <Max> entry defining the value's range.

<Integer Name="Gain">

<pValue>GainReg</pValue>

<Min>0</Min>

<Max>100</Max>

</Integer>

<IntReg Name="GainReg" >

<Address>0xffff0000</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Endianess>LittleEndian</Endianess>

</IntReg>

<Port Name="Device"/>

As the graphic shows we are dealing now with three nodes linked together:

In order to access the minimum and maximum via the API use the GetMin and GetMax methods if the IInteger interface as shown in the example:

CIntegerPtr ptrGain = Camera._GetNode("Gain");

ptrGain->SetValue(42);

cout << "Gain.Value = " << ptrGain->GetValue() << "\n";

cout << "Gain.Min   = " << ptrGain->GetMin() << "\n";

cout << "Gain.Max   = " << ptrGain->GetMax() << "\n";

Note that ptrGain needs to be connected to the node with the name Gain. If you run the example the output is

Gain.Value = 42

Gain.Min   = 0

Gain.Max   = 100

You may wonder what the minimum and maximum of the GainReg node is. Just extend the code and have a look:

CIntegerPtr ptrGainReg = Camera._GetNode("GainReg");

cout << "GainReg.Value = " << ptrGainReg->GetValue() << "\n";

cout << "GainReg.Min   = " << ptrGainReg->GetMin() << "\n";

cout << "GainReg.Max   = " << ptrGainReg->GetMax() << "\n";

The result is:

GainReg.Value = 42

GainReg.Min   = 0

GainReg.Max   = 4294967295

This makes sense if you think about the definition of GainReg: it is an unsigned 4 byte integer which indeed has a range of 0 to 232-1=4294967295. So the Gain node just restricted the range of the GainReg node.

How do you know which nodes exist and which elements they take? The syntax of a GenApi XML file is defined through a XSD schema file GenApiSchema_Version_1_1.xsd found in the subdirectory /xml/GenApi. For better readability the schema is transformed into a HTML document GenApiSchema_Version_1_1.html. In this document the full documentation of the <IntReg> node looks like this:

Hint: [1] = mandatory, [0..1] = optional, [0..*] = any number

Of course the standard text itself is inevitable in order to understand the meaning of the different elements.

The schema is also very useful because XML editors like the one built into VisualStudio use it to provide Intellisense support like this:

The schema enforces a defined sequence for the node entries and Intellisense gives you a list of what could be inserted next. If your XML code is not syntactically correct VisualStudio will underline the statement with the problem and give you a hint what's wrong. In the example at least an <AccessMode> element is missing.

## Minimum and Maximum Extended with Logging

In reality you would try to avoid hard coding the minimum and maximum to an XML file as shown in the previous example but instead fetch the values from the camera at runtime. Let us assume the following register layout:

| Register | Address | Length |
| --- | --- | --- |
| GainValue | 0xffff0000 | 4 |
| GainMin | 0xffff0004 | 4 |
| GainMax | 0xffff0008 | 4 |

In order to map this we use the following XML file:

<Integer Name="Gain">

<pValue>GainValueReg</pValue>

<pMin>GainMinReg</pMin>

<pMax>GainMaxReg</pMax>

</Integer>

<IntReg Name="GainValueReg" >

<Address>0xffff0000</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Endianess>LittleEndian</Endianess>

</IntReg>

<IntReg Name="GainMinReg" >

<Address>0xffff0004</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Endianess>LittleEndian</Endianess>

</IntReg>

<IntReg Name="GainMaxReg" >

<Address>0xffff0008</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Endianess>LittleEndian</Endianess>

</IntReg>

<Port Name="Device"/>

The graphical node layout looks like this:

As you can see we have now three <IntReg> nodes, one for the value, one for the minimum and one for the maximum. All three feed to the <Integer> node Gain and connect to the <Port> node Device.

For providing a pseudo implementation of the three registers we are using a helper class CStructPort<class DataStruct> which implements an IPort interface accessing the members of a struct holding the actual data. Note that in order to make sure the compiler does not spoil the expected memory layout we need to a #pragma pack statement. When defining the CameraRegsiters variable the BaseAddress is given as parameter to the constructor. Since the instantiated CStructPort template inherits from the struct CameraRegsiters_t given as template parameter you can directly access and initialize the struct's values.

#   pragma pack(push, 1)

typedef struct

{                           // addr:len

uint32_t GainValue;     // 0:4

uint32_t GainMin;       // 4:4

uint32_t GainMax;       // 8:4

} CameraRegisters_t;        // 4

#   pragma pack(pop)

CTestPortStruct<CameraRegisters_t> CameraRegisters( 0xffff0000 );

CameraRegisters.GainValue = 42;

CameraRegisters.GainMin = 0;

CameraRegisters.GainMax = 100;

Camera._Connect( &CameraRegisters, "Device" );

CIntegerPtr ptrGain = Camera._GetNode("Gain");

ptrGain->SetValue(42);

cout << "Gain.Value = " << ptrGain->GetValue() << "\n";

cout << "Gain.Min   = " << ptrGain->GetMin() << "\n";

cout << "Gain.Max   = " << ptrGain->GetMax() << "\n";

The output of this example is the same as for the previous one. However this XML file is now independent from the camera's actual minimum and maximum values as long as the register layout matches.

If node maps get more complicated debugging can become quite a problem. In order to support this the GenICam reference implementation contains a quite sophisticated logging system. Open the DebugLogging.properties file in the /log/config subdirectory and enable the value logging taking away the comment sign (the hash '#') from the line log4j.category.GenApi.Device.Value=DEBUG, Win32Debug.

If you now run the GettingStarted::MiniMaxReloaded example under the debugger you will get the following additional debug output to your Output window when executing the line ptrGain->SetValue(42):

=>INFO : GenApi.Device.Value.Gain : SetValue( 42 )...

=>INFO   : GenApi.Device.Value.GainMinReg : GetValue...

=>INFO      : GenApi.Device.Value.GainMinReg : Get...

=>INFO         : GenApi.Device.Value.GainMinReg : ...Get( 4 ) = 0x00000000

=>INFO      : GenApi.Device.Value.GainMinReg : ...GetValue = 0

=>INFO   : GenApi.Device.Value.GainMaxReg : GetValue...

=>INFO      : GenApi.Device.Value.GainMaxReg : Get...

=>INFO         : GenApi.Device.Value.GainMaxReg : ...Get( 4 ) = 0x64000000

=>INFO      : GenApi.Device.Value.GainMaxReg : ...GetValue = 100

=>INFO   : GenApi.Device.Value.GainMinReg : GetValue = 0  (from cache)

=>INFO   : GenApi.Device.Value.GainValueReg : SetValue( 42 )...

=>INFO      : GenApi.Device.Value.GainValueReg : Set( 4, 0x2A000000 )...

=>INFO         : GenApi.Device.Value.GainValueReg : ...Set

=>INFO      : GenApi.Device.Value.GainValueReg : ...SetValue

=>INFO   : GenApi.Device.Value.Gain : ...SetValue

=>INFO : GenApi.Device.Value.Gain : GetValue = 42  (from cache)

=>INFO : GenApi.Device.Value.GainMinReg : GetValue = 0  (from cache)

=>INFO : GenApi.Device.Value.GainMaxReg : GetValue = 100  (from cache)

Let us analyse the sequence: It starts by the Gain.SetValue( 42) statement followed by two GetValue statements, one for GainMinReg and one for GainMaxReg. This is required because GenICam needs to check if the value 42 to be set lies between the minimum value 0 and the maximum value 100. After that is granted the line GainValueReg.Set( 4, 0x2A000000 ) indicates the actual register access. The last three lines correspond to the three cout << statements. Note that GenICam takes the requested values from its internal cache in order to minimize the number of camera access operations.

You can enable logging for different aspects of GenICam, e.g. the aspect how the AccessMode is determined and you can restrict the logging to specific nodes which comes in handy for large node trees. For details see HowTo_use_the_Logging.doc.

## Dealing with Bit Fields, Enumerations, and Commands

Often multiple features are packed into one integer. In order to demonstrate how GenICam deals with that situation we extend the Gain related register block as shown in the table below.

| Register | Field | Address | Length | Bits | Comment |
| --- | --- | --- | --- | --- | --- |
| GainValue | - | 0xffff0000 | 4 | [0..31] |  |
| GainMin | - | 0xffff0004 | 4 | [0..31] |  |
| GainMax | - | 0xffff0008 | 4 | [0..31] |  |
| GainCtrl | - | 0xffff000C | 4 | - |  |
| " | GainPresenceInq | " | - | [0] | 1 if Gain feature is present |
| " | GainAuto | " | - | [1..2] | Automated gain control 0 : Off 1 : Once 2 : Continuous |
| " | GainOnePush | " | - | [3] | Writing 1 will trigger one round of automated gain control |
| " | Reserved | " | - | [4..31] |  |

In order to extract the GainPresenceInq bit we use a <MaskedIntReg> node as shown below. Its syntax is quite similar to the <IntReg> but it can have an additional <Bit> entry indicating which bit to extract.

<Integer Name="Gain">

<pIsImplemented>GainPresenceInqReq</pIsImplemented>

<pValue>GainValueReg</pValue>

<pMin>GainMinReg</pMin>

<pMax>GainMaxReg</pMax>

</Integer>

<MaskedIntReg Name="GainPresenceInqReq">

<Address>0xffff000C</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Bit>0</Bit>

<Endianess>LittleEndian</Endianess>

</MaskedIntReg>

In order to tell GenICam that the Gain is implemented only if the GainPresenceInq is set <pIsImplemented> link was added to the Gain node pointing to GainPresenceInq. Client code should always be aware of the possibility that a feature might not be implemented so the code accessing the Gain should always look like shown below: before touching a feature always check its AccessMode first. If you don't and access a feature which is by chance not implemented GenICam will throw an exception.

CIntegerPtr ptrGain = Camera._GetNode("Gain");

if( RW == ptrGain->GetAccessMode() )

{

ptrGain->SetValue(42);

cout << "Gain.Value = " << ptrGain->GetValue() << "\n";

cout << "Gain.Min   = " << ptrGain->GetMin() << "\n";

cout << "Gain.Max   = " << ptrGain->GetMax() << "\n";

}

The GainAuto register is two bits wide and is also accessed via a <MaskedIntReg> node GainAutoReg using a pair of <MSB> and <LSB> entries. This yields an integer but in fact we want GainAuto to be defined as an enumeration so we add a <Enumeration> node GainAuto which is linked to GainAutoReg and gives symbolic names to the values Off=0, Once=1, and Continuous=2.

<Enumeration Name="GainAuto">

<EnumEntry Name="Off">

<Value>0</Value>

</EnumEntry>

<EnumEntry Name="Once">

<Value>1</Value>

</EnumEntry>

<EnumEntry Name="Continuous">

<Value>2</Value>

</EnumEntry>

<pValue>GainAutoReq</pValue>

</Enumeration>

<MaskedIntReg Name="GainAutoReq">

<Address>0xffff000C</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<LSB>1</LSB>

<MSB>2</MSB>

<Endianess>LittleEndian</Endianess>

</MaskedIntReg>

The symbolic values are used when an enumeration feature is converted to and from string representation as shown in the following code example:

CEnumerationPtr ptrGainAuto = Camera._GetNode("GainAuto");

ptrGainAuto->FromString("Continuous");

cout << "GainAuto.Value = " << ptrGainAuto->ToString() << "\n";

The last register GainOnePush issues a command when writen a command value 1 to the register. In GenICam this is mapped using a <Command> node which writes the CommandValue to the GainOnePushReg:

<Group Comment="GainOnePush">

<Command Name="GainOnePush">

<pValue>GainOnePushReg</pValue>

<CommandValue>1</CommandValue>

</Command>

<MaskedIntReg Name="GainOnePushReg">

<Address>0xffff000C</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Bit>3</Bit>

<Endianess>LittleEndian</Endianess>

</MaskedIntReg>

</Group>

The code to use a command looks like this:

CCommandPtr ptrGainOnePush = Camera._GetNode("GainOnePush");

ptrGainOnePush->Execute();

If you look at the XML code above you may have notices that the two nodes GainOnePush and GainOnePushReg are living inside a <Group> bracket. This bracket has no syntactical meaning on its own and is just there so structure the XML code and support the outlining feature most XML editors have like, e.g. VisualStudio:

## XML Formulas

Sometimes the values retrieved from the raw register must be processed in order to yield the required user experience. This is especially true for older cameras whose register layout has not been designed with GenICam in mind and which are now expected to behave like a proper SFNC camera through GenICam magic.

For these kind of magic GenICam provides a node type with an embedded formula interpreter known as the SwissKnife. To demonstrate its usage we add two more registers to our camera's register space as shown below:

| Register | Field | Address | Length | Bits | Comment |
| --- | --- | --- | --- | --- | --- |
| ... | ... | ... | ... | ... |  |
| ExposureTick | - | 0xffff0010 | 4 | [0..31] | Exposure time in ticks |
| ExposureTimeBase | - | 0xffff0014 | 8 | [0..31] | double value indicating the length of one tick in [s] |

The task is to provide a float node ExposureTime giving access to the time in [s] even if no direct register exists. Mathematically you need two transformations, one for reading the ExposureTime value and one for writing:

For Reading:	ExposureTime := ExposureTick * ExposureTimeBase

For Writing:	ExposureTick := ExposureTime / ExposureTimeBase

As shown in the following graph ExposureTick is mapped via an <IntReg> whereas ExposureTimeBase is mapped by an <FloatReg>. The two formulas are evaluated within a <Converter> node which provides the required float interface for the ExposureTime feature.

The XML code is shown below. The <Converter> node primarily connects the ExposureTimeTicksReg because that the node it writes to. The ExposureTimeBaseReg is read only. The <pVariable> entry is used to give it the name BASE which references it in the two formulas. In order to understand the To/From notation think of yourself looking "from above" to the node converter: the <FormulaTo> describes the operation required when writing 'to' the underlying node whereas the <FormulaFrom> does the opposite. The variables TO and FROM are implicitly declared and denote the converter's own value (FROM) and the underlying node's value (TO) respectively.

<Converter Name="ExposureTime">

<pVariable Name="BASE">ExposureTimeBaseReg</pVariable>

<FormulaTo>FROM / BASE</FormulaTo>

<FormulaFrom>TO * BASE</FormulaFrom>

<pValue>ExposureTicksReg</pValue>

<Unit>s</Unit>

</Converter>

<IntReg Name="ExposureTicksReg" >

<Address>0xffff0010</Address>

<Length>4</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Endianess>LittleEndian</Endianess>

</IntReg>

<FloatReg Name="ExposureTimeBaseReg" >

<Address>0xffff0014</Address>

<Length>8</Length>

<AccessMode>RW</AccessMode>

<pPort>Device</pPort>

<Endianess>LittleEndian</Endianess>

</FloatReg>

<Port Name="Device"/>

Running the test code

CFloatPtr ptrExposureTime = Camera._GetNode("ExposureTime");

ptrExposureTime->SetValue(42E-6);

cout << "ExposureTime  = " << ptrExposureTime->GetValue() << " [" <<

ptrExposureTime->GetUnit() << "]\n";

cout << "ExposureTicks = " << CameraRegisters.ExposureTicks << "\n";

then yields the output:

ExposureTime  = 4.2e-005 [s]

ExposureTicks = 42

Note the unit [s] which has been provided by the <Unit> element of the ExposureTime node and retrieved by the node's GetUnit method.

Just to give you an idea how powerful the SwissKnife is here the list of supported operators:

- ( )	brackets

- + - * / 	addition, subtraction, multiplication, division

- % 	remainder

- ** 	power

- & | ^ ~ 	bitwise and / or / xor / not

- <> = > < <= >=	logical relations not equal / equal / greater / less / 
less of equal / greater or equal

- && ||	logical and / or

- << >>	shift left, shift right

- Conditional operator:

- <condition> ? <true expr.> : <false expr.>

- Functions:

- SGN, NEG,

- Functions present only with the SwissKnife but not with the IntSwissKnife:

- ATAN, COS, SIN, TAN, ABS, EXP, LN, LG, SQRT,

- TRUNC, FLOOR, CEIL, ROUND( x, precision ),

- ASIN, ACOS, SGN, NEG, E, PI

When embedding formulas in XML files the problem arises that the characters <,  >, and & cannot be used directly because they are part of the XML syntax. There are two possible solutions for that problem.

First you can escape these letters as follows:

- <	becomes	&lt;	(lt = thess than)

- >	becomes 	&gt;	(gt = greater than)

- &	becomes	&amp;	(amp = ampersand)

As a result the formula (x>0) && (x<10) becomes

<formula>(x &gt; 0) &amp;&amp; (x &lt; 10)</formula>

Alternatively you can declare the whole formula as non-XML-text by bracketing it with <![CDATA[ and ]]>. The formula then becomes:

<formula><![CDATA[ (x>0) && (x<10)  ]]>/formula>

## The Feature Tree and Callbacks

As you may imagine an elaborated camera device can have thousands of nodes in its XML files, most of them being of no interest to the customer. In fact only those nodes which describe features should be exposed to the outside. GenICam supports this via <Category> nodes which form a tree starting at the category with the SFNC defined name Root. The following example uses all nodes from the previous sections which are collected into one XML file.

<Category Name="Root">

<pFeature>AnalogControls</pFeature>

<pFeature>AcquisitionControl</pFeature>

</Category>

<Category Name="AnalogControls">

<pFeature>Gain</pFeature>

<pFeature>GainAuto</pFeature>

<pFeature>GainOnePush</pFeature>

</Category>

<Category Name="AcquisitionControl">

<pFeature>ExposureTime</pFeature>

<pFeature>ExposureTicks</pFeature>

</Category>

The tree can be recursively dumped using the following function

void DumpFeatures( const CNodePtr &ptrFeature, const int Indent = 0 )

{

string IndentStr( Indent, ' ');

CCategoryPtr ptrCategory = ptrFeature;

if( ptrCategory.IsValid() )

{

cout << IndentStr << "Category '" << ptrFeature->GetName() << "'\n";

FeatureList_t Features;

ptrCategory->GetFeatures(Features);

for( FeatureList_t::iterator itFeature=Features.begin();

itFeature!=Features.end(); itFeature++ )

DumpFeatures( (*itFeature), Indent + 2 );

}

else

cout << IndentStr << "'" << ptrFeature->GetName() << "'\n";

}

which is called like this:

CNodePtr ptrRoot = Camera._GetNode("Root");

cout << "Dumping Feature Tree:\n";

DumpFeatures( ptrRoot, 2 );

and yields the following output:

Dumping Feature Tree:

Category 'Root'

Category 'AnalogControls'

'Gain'

'GainAuto'

'GainOnePush'

Category 'AcquisitionControl'

'ExposureTime'

'ExposureTicks'

Note that the categories have no influence on the behaviour of the nodes they refer to. They are just for indication which nodes are so called Feature Nodes and which are Hidden or Implementation Nodes. Every node map for a real camera should have at least a Category::Root and a Port::Device node: this first one is expected by any viewer to be able to show the feature tree and the latter one is required to connect the transport layers.

If you look at the category AcquisitionControl you see that both the ExposureTicks and the ExposureTime nodes have been defined as feature. Since ExposureTime depends on ExposureTicks the following problem arises: Think of a viewer which displays both features' values. If the user changes, say ExposureTicks this will also change the value of ExposureTime. But how exactly does the viewer know it has to update its display of ExposureTime? Since this is a very common problem GenICam supports the model view controller (MVC) pattern: you can register a callback on every node which will fire if the node's value gets invalidated. In the viewer example this means the viewer has to register a callback to every node and refresh the node's display every time the callback fires. Note that a fireing callback does not mean the node's value has changed – it just means it may have changed.

Below an example for a callback function is shown which gets a pointer pNode to the changed node as argument so you can if you like register the same function to multiple nodes and decide based on the actual value of pNode what to do with the callback.

void OnChanged( INode *pNode )

{

cout << "The node '" << pNode->GetName() << "' has been invalidated\n";

CValuePtr ptrValue = pNode;

if( ptrValue.IsValid() )

cout << "Value = " << ptrValue->ToString() << "\n";

}

The following code registers the callback on the ExposureTime node and then writes a value to the ExposureTicks node.

CFloatPtr ptrExposureTime = Camera._GetNode("ExposureTime");

CIntegerPtr ptrExposureTicks = Camera._GetNode("ExposureTicks");

Register( ptrExposureTime->GetNode(), &OnChanged );

cout << "Now writing to ExposureTicks\n";

*ptrExposureTicks = 4711;

The resulting output is shown below: the callback fires as soon as the ExposureTicks node has been written because this changes also the ExposureTime value.

Now writing to ExposureTicks

The node 'ExposureTime' has been invalidated

Value = 0.004711

## The Static Use Case

Up to now we have been always fetching nodes by name using the Camera._GetNode(“MyNode”) method. While this is fine for a completely generic program it can become quite tedious and therefore GenICam provides a way to create a static camera pointer which provides full Intellisense support for all a camera’s features.

The static pointer is created from the camera’s XML file by calling the GenApiPreProcessor tool which comes with GenICam. The static pointer requires two header files which are created like this:

GenApiPreProcessor.exe –x MyCamera.xml –s GenApi_Params_h.xsl –o MyCameraParams.h

GenApiPreProcessor.exe –x MyCamera.xml –s GenApi_Ptr_h.xsl –o MyCamera.h

By including MyCamera.h the static pointer is made available. It lives in a name space composed as <VendorName>_<ModelName>, is named C<ModelName>, and inherits from CNodeMapRef so you can just load the XML file and connect to the port as usual.

using namespace GenApiTutorial_GettingStarted_StaticUseCase;

CGettingStarted_StaticUseCase Camera;

Camera._LoadXMLFromFile("GenApiTutorial", "GettingStarted_StaticUseCase");

Camera._Connect( &CameraRegisters, "Device" );

The big difference to CNodeMapRef is that the created class contains embedded pointer references for all features from the XML file (i.e. all nodes which are directly reachable from the Root category via sub-categories). Here’s the Intellisense list showing all features of our test camera:

With the embedded features it is now much easier to write code as shown below. Note that the enum values are mapped to C++ enumerations composed as <FeatureName>_<SymbolicName>.

Camera.GainAuto = GainAuto_Continuous;

Camera.Gain = 42;

Camera.GainAuto = GainAuto_Once;

Camera.GainOnePush();

cout << "ExposureTime = " << Camera.ExposureTime() << "\n";

You may wander what happens if the XML file loaded does not match the XML from which the static pointer war originally created. However this does not matter: if a new node is added to the XML file loaded the static pointer does not know about the new node is simply ignored. And it a node is not present in the XML file the corresponding static pointer feature just returns NotImplemented as AccessMode.

For details about the supported Compilers see the Building_and_Releasing.doc.

If you had VisualStudio running while installing GenICam you need to re-start the studio so the new environment variables can take effect.

Note that V2_1 is the current GenICam version and will increase over time.

You can copy the file from $(GENICAM_ROOT)/Tests/GenApi/HelloWorld

The reason is that the test code relies on the CPPUnit framework which in turn uses STL strings in it public interface. As are result you need to use the same compiler for the client code and all DLLs.

These kind or graphics are automatically created by the Snipper using the DOT tool from the GraphViz suite.

Unsigned is the default for <IntReg> nodes but you can of course also have signed integers.

Currently the classic v1.0 schema and an extended v1.1 introduced with GenICam v2.0 exist. The schema v1.1 is backward compatible to v1.0 and the reference implementation can process them both. This document refers to v1.1.

In most cases the sequence does not bear a meaning but unfortunately the XSD schema language is not powerful enough to deal with an arbitrary sequence so the decision was either live with strict sequence or use the Intellisense support...

A notorious example is the IIDC v1.31 spec which can be transformed to SFNC behavior even if the resulting XML file looks a little more complicated than usual.